# Beginner's Tutorial: The MATLAB Path of the Streaming Range-Doppler SAR Repository

Table of Contents

# Beginner's Tutorial: The MATLAB Path of the Streaming Range-Doppler SAR Repository

This tutorial walks through the MATLAB part of this repository from zero. You do not need to know SAR or FFT hardware design beforehand. The Simulink model sys1.slx is out of scope here.

## 0. The 60-second picture

A SAR (Synthetic Aperture Radar) flies past a scene and sends many radar pulses. Each pulse's echo is one column of a 2-D table:

pulse 1  pulse 2  ...  pulse 512     <- "azimuth" (slow time, along the flight path)
sample 1      .         .             .
sample 2      .         .             .
  ...
sample 1024   .         .             .
   ^
   "range" (fast time, distance from the radar)

The raw table looks like noise. To turn it into an image of two bright dots (the two targets) we do two compressions, each one an FFT, multiply, IFFT sequence:

| Step | Direction | Operation | Purpose |
| --- | --- | --- | --- |
| 1 | down each column (range) | FFT | go to frequency domain |
| 2 | down each column | multiply by range filter H_ref | matched filter for the chirp |
| 3 | down each column | IFFT | range compression done |
| 4 | along each row (azimuth) | FFT | go to Doppler domain |
| 5 | along each row | multiply by azimuth filter | matched filter for the Doppler chirp |
| 6 | along each row | IFFT | azimuth compression done → image |

This is the Range-Doppler Algorithm (RDA), minus a refinement called range-cell-migration correction. For these parameters the correction isn't needed (see §3.3).

The repo does steps 1–6 twice:

- Classic path uses MATLAB's built-in fft / ifft. This is the reference answer.

- Proposed (streaming) path replaces the azimuth FFT/IFFT with hand-written, stage-by-stage FFTs that mimic how an FPGA would do it. It produces the same image, or should (see §6, where I found a catch).

## 1. The file map

| File | Role | Type |
| --- | --- | --- |
| init_param.m | Builds the simulated radar data and all the filters | function |
| main.m | The script you run. Runs the classic path, then the streaming path, then plots | script |
| stream_fft.m | A 512-point FFT made of 9 chained DIF stages | function |
| fft_dif.m | One stage of a Decimation-In-Frequency FFT | function |
| stream_fft_dit.m | A 512-point FFT made of 9 chained DIT stages | function |
| fft_dit.m | One stage of a Decimation-In-Time FFT | function |
| sys1.slx | Simulink / System Generator model (not covered) | model |

Call hierarchy:

main.m
 ├── init_param.m                (makes rawData, H_ref, H_az1, H_az2)
 ├── stream_fft.m  ── fft_dif.m  (called 9 times, stage 9 down to 1)
 └── stream_fft_dit.m ─ fft_dit.m (called 9 times, stage 1 up to 9)

To run it: open MATLAB in the repo folder, type main, press Enter. You need MATLAB plus the Signal Processing Toolbox (for bitrevorder).

## 2. MATLAB basics you need first

Skim this if you already know MATLAB.

| Syntax | Meaning |
| --- | --- |
| clc; clear; | clear the console and all variables |
| % / %% | comment / start of a "section" you can run separately |
| A(:,i) | the whole column i of matrix A |
| A(i,:) | the whole row i of matrix A |
| size(A,1), size(A,2) | number of rows, number of columns |
| zeros(size(A)) | pre-allocate a matrix of zeros the same shape as A |
| 1j | the imaginary unit (radar data is complex) |
| .* | element-by-element multiply (* would be matrix multiply) |
| .' | plain transpose (flips rows↔columns, no conjugate) |
| ' | transpose with conjugate |
| abs(z) | magnitude of complex numbers |
| [a,b] = f(x) | call a function that returns two outputs |
| function [out1,out2] = name(in) | defines a function; the file name must match |
| fft(v) / ifft(V) | built-in Fourier transform and inverse |

Why FFT at all? Convolving a signal with a filter (expensive) equals multiplying their spectra (cheap): ifft( fft(x) .* H ). Both compressions use exactly this trick.

## 3. init_param.m: building the fake radar data

This function returns four things: rawData, H_ref, H_az1, H_az2.

### 3.1 Parameters (lines 3–14)

| Variable | Value | Meaning |
| --- | --- | --- |
| fc | 10 GHz | carrier frequency (X-band) |
| c, lambda | 3e8, c/fc = 3 cm | speed of light, wavelength |
| fs | 100 MHz | sample rate of the receiver (ADC) |
| prf | 1000 Hz | pulses per second |
| Tp | 1 µs | chirp duration |
| B | 50 MHz | chirp bandwidth |
| K | B/Tp = 5e13 | chirp rate (Hz per second) |
| v | 100 m/s | platform speed |
| numPulses | 512 | columns (azimuth) |
| numSamples | 1024 | rows (range) |
| targets | [1000 0; 1200 50] | two point targets: (range 1000 m, azimuth 0 m) and (range 1200 m, azimuth 50 m) |

Quick sanity numbers you can derive:

- One range sample = c/(2*fs) = 1.5 m (that's alphaR, line 38).

- One pulse spacing along the track = v/prf = 0.1 m.

- A chirp lasts Tp*fs = 100 samples.

### 3.2 The transmitted chirp (lines 15–17)

t = (0:1/fs:Tp-1/fs);
txChirp = exp(1j*pi*K*t.^2);

A chirp is a pulse whose frequency sweeps linearly. Its phase is πKt². Sweeping a wide band is what lets a radar tell two nearby targets apart after compression.

### 3.3 Simulating the raw echoes (lines 18–33)

for p = 1:numPulses                           % each pulse = one column
    azPos = (p - numPulses/2) * v / prf;      % where the radar is along its path
    for k = 1:size(targets,1)                 % each target
        R   = sqrt(targets(k,1)^2 + (azPos - targets(k,2))^2);  % distance radar↔target
        tau = 2 * R / c;                      % round-trip time
        idx = round(tau * fs) + 1;            % which sample the echo starts at
        ...
            phase = exp(-1j * 4 * pi * fc * R / c);   % carrier phase from distance
            rawData(idx:idx+99, p) += phase * txChirp.';  % drop a delayed chirp in

In words: for each pulse and each target, compute the distance, convert it to a delay, and place a copy of the chirp at that delay, multiplied by a phase exp(-j4πfcR/c). That phase changing from pulse to pulse is the Doppler signal. It is what azimuth compression later exploits. The if on line 26 skips echoes that would run off the end of the 1024 samples.

Line 33 adds a little complex Gaussian noise (0.01*randn). Because of the noise, each run produces slightly different data.

Why no range-cell-migration correction? Over the aperture, R for the first target changes by only about sqrt(1000²+25.6²) − 1000 ≈ 0.33 m, which is far smaller than one 1.5 m range cell. The echo therefore stays in the same row, so each row can be treated independently. (This is my own check from the numbers, not a claim made in the repo.)

### 3.4 The range filter H_ref (line 35)

H_ref = fft(conj(fliplr(txChirp)), numSamples);

- fliplr reverses the chirp in time, conj conjugates it. That is the matched filter for the chirp.

- fft(..., numSamples) zero-pads it to 1024 points and takes its spectrum.

- The result is a 1×1024 vector. Multiplying a column's spectrum by it is the same as correlating the column with the chirp.

### 3.5 The azimuth filters H_az1, H_az2 (lines 36–47)

fd = (0:numPulses-1) * (prf / numPulses);   % Doppler frequency axis 0 .. ~prf
fd(fd > prf/2) = fd(fd > prf/2) - prf;      % wrap the upper half to negative frequencies
R0 = (1:numSamples) * alphaR;               % range (m) of each row
Ka = -2 * v^2 ./ (lambda * R0);             % Doppler chirp rate; depends on range
h  = exp(-1j * pi * (fd.^2) / Ka(r));       % azimuth matched filter for row r

- Along the flight path the Doppler signal is itself a chirp, and its rate Ka depends on range, so every row gets its own filter. H_az1 is therefore a 1024×512 matrix.

- fd is arranged in the same order MATLAB's fft outputs (0, positive, then negative frequencies).

The key difference:

H_az1(r,:) = h;                      % natural frequency order  -> classic path
H_az2(r,:) = h(bitrevorder(1:512));  % bit-reversed order       -> streaming path

bitrevorder(1:512) returns the indices 1..512 in bit-reversed order. Why the streaming path needs this is explained in §5.

## 4. main.m, part 1: the classic path (lines 3–35)

[rawData,H_ref,H_az1,H_az2] = init_param;   % build everything

| Lines | Variable | What it does |
| --- | --- | --- |
| 5–8 | x1 | FFT of every column (range FFT) |
| 9–13 | x2 | multiply each column by H_ref.' (.' turns the 1×1024 row into a 1024×1 column) |
| 14–18 | x3 | IFFT of every column → range-compressed data |
| 19–23 | x4 | FFT of every row (azimuth FFT) |
| 24–28 | x5 | multiply row i by its own filter H_az1(i,:) |
| 29–33 | x6 | IFFT of every row → azimuth-compressed data |
| 35 | x7 = abs(x6) | magnitude, which is the image |

The pattern to learn: for i = 1:size(A,2) + A(:,i) loops over columns; for i = 1:size(A,1) + A(i,:) loops over rows. (MATLAB's fft(A,[],1) would do all columns at once, but the author loops explicitly to mirror a hardware pipeline.)

## 5. The streaming path: FFTs you can build in hardware

### 5.1 Why not just call fft?

On an FPGA there is no fft() function. You build the FFT from small identical stages ("butterflies"). A 512-point FFT has log₂512 = 9 stages, and data flows through them like water through pipes, which is the "streaming" idea. The repo writes each stage as its own MATLAB function so the Simulink model can be checked against it stage by stage.

### 5.2 One butterfly stage

Every stage processes pairs of values (u, v) that are m/2 apart and writes two outputs. Here m = 2^stage, and wm = exp(-j2π/m) generates the twiddle factors w = 1, wm, wm², ….

fft_dif.m (Decimation In Frequency): multiply after the add/subtract.

y(k+j+1)       = u + v;
y(k+j+1+m/2)   = (u - v) * w;

fft_dit.m (Decimation In Time): multiply before the add/subtract.

v = x(k+j+1+m/2) * w;
y(k+j+1)       = u + v;
y(k+j+1+m/2)   = u - v;

The loops:

- for k = 0:m:512-1: walk through the data in blocks of size m.

- for j = 0:(m/2-1): walk through the pairs inside a block.

- ww(j+1) = w; records the twiddle values used, so you can compare them with the hardware's twiddle ROM.

Beginner note: the +1 everywhere is because MATLAB arrays start at index 1, not 0. Also, both functions hard-code 512, so they only work for 512-point data.

### 5.3 Chaining stages

stream_fft.m calls fft_dif for stages 9, 8, 7 … 1:

[y9,w9] = fft_dif(xx,9);   % blocks of 512
[y8,w8] = fft_dif(y9,8);   % blocks of 256
...
[y1,w1] = fft_dif(y2,1);   % blocks of 2

stream_fft_dit.m does the mirror image, stages 1 → 9.

### 5.4 The bit-reversal trick (the clever part)

- A DIF FFT takes input in natural order and produces output in bit-reversed order.

- A DIT FFT takes input in bit-reversed order and produces output in natural order.

So if you chain DIF → (point-wise multiply) → DIT, no reordering memory is needed in between. That saves a lot of RAM on an FPGA. The only price: the multiplication happens on bit-reversed data, so the filter must be bit-reversed as well. That is exactly why H_az2 = h(bitrevorder(1:512)) exists.

I verified numerically (Python port of these two files) that the 9 DIF stages produce exactly fft(x) in bit-reversed order.

### 5.5 main.m, part 2 (lines 36–68)

| Lines | Variable | What it does |
| --- | --- | --- |
| 38–41 | xx1 | range FFT (same as classic) |
| 42–46 | xx2 | × H_ref.' (same) |
| 47–51 | xx3 | range IFFT (same) |
| 52–56 | xx4 | azimuth FFT via stream_fft (9 DIF stages), output bit-reversed. The w1…w9 outputs are the twiddle tables, overwritten each row |
| 57–61 | xx5 | × H_az2 (bit-reversed filter) |
| 62–66 | xx6 | stream_fft_dit (9 DIT stages), back to natural order |
| 68 | xx7 = abs(xx6) | magnitude |

### 5.6 Plotting (lines 70–73)

subplot(2,1,1); mesh(x7)          % classic image, top
subplot(2,1,2); mesh(xx7/512)     % streaming image, bottom, divided by 512

The /512 is there because an un-normalised FFT scales its result by N = 512, while MATLAB's ifft includes the 1/N. Two peaks should appear in each surface plot, one for each target.

## 6. A catch worth knowing about (my finding, please verify on your MATLAB)

The classic path ends with an IFFT (ifft). The streaming path ends with fft_dit, which uses the forward twiddle exp(-j2π/m). That is a forward FFT, not an inverse one.

A forward FFT applied where an inverse FFT belongs gives N · x[(−n) mod N]. That is the right answer, scaled by N and index-reversed along azimuth. Dividing by 512 fixes the scale. Taking abs hides the phase, but the reversal remains.

I checked this in a Python port: first on random data with a stand-in filter, then on a port of the full pipeline (init_param + main), where the flipped comparison matched to a relative error of about 2e-16, versus about 0.59 unflipped:

DIF out == bit-reversed FFT:                True
stream |out|/N == |classical|:              False
stream |out|/N == index-reversed classical: True

What this means for you:

- The two mesh plots may show the azimuth axis mirrored relative to each other. A mirrored image of a target at a different position would look like a shifted target.

- To compare them numerically, flip one of them, e.g. compare x7(:, [1 512:-1:2]) against xx7/512.

- A proper fix would be to conjugate the twiddle (use exp(+1j*2*pi/m)) in fft_dit.m and divide by 512, or to conjugate the input and output of the last FFT.

Caveat: MATLAB is not installed in my environment, so I did not run main.m itself; the Python port stands in for it. Run main and compare the two plots to confirm.

## 7. Hands-on exercises

- Run it. Type main. Both surfaces should show two peaks. Look for them near rows 766 and 900. The echo starts at row ≈ 2*1000/3e8*100e6 + 1 ≈ 668, but the matched filter output peaks about one chirp length (≈ 99 samples) later. Likewise for the second target (starts ≈ 801).

- Numerical comparison. After running, execute:

- err1 = norm(x7 - xx7/512, 'fro') / norm(x7, 'fro')
xr   = xx7(:, [1 512:-1:2]) / 512;
err2 = norm(x7 - xr, 'fro') / norm(x7, 'fro')

- If §6 is right, err2 should be far smaller than err1.

- Test one stage chain alone.

- x = randn(1,512) + 1j*randn(1,512);
[y,~] = stream_fft(x);
max(abs(y - fft(x)(bitrevorder(1:512))))   % should be ~1e-12

- (In older MATLAB write it with a temporary variable, since you cannot index a function result directly.)

- Remove a target. Change targets in init_param.m to a single row. Re-run and watch one peak disappear.

- Move a target in range. Change 1000 to 1100. Does the peak move by about 67 rows? (100 m ÷ 1.5 m.)

- Make the streaming path match. Apply the fix from §6 and rerun exercise 2.

- Speed. Time the classic path versus the streaming path with tic; ...; toc. Why is the hand-written one so much slower in MATLAB? (Hint: nested loops, and y = zeros(1,m) grows inside the loop in the early stages.)

## 8. Glossary

| Term | Plain meaning |
| --- | --- |
| Range / fast time | distance from the radar, measured along each received pulse |
| Azimuth / slow time | position along the flight path, measured pulse to pulse |
| Chirp | pulse whose frequency sweeps linearly |
| Matched filter | the filter that gives the strongest output peak for a known signal |
| Pulse compression | turning a long chirp echo into a sharp peak |
| Doppler | frequency shift caused by relative motion; in SAR it varies along the flight path |
| PRF | pulse repetition frequency, i.e. how often pulses are sent |
| Butterfly | the basic add/subtract pair operation inside an FFT |
| Twiddle factor | the complex rotation exp(-j2πk/m) used between FFT stages |
| DIF / DIT | two equivalent FFT structures; one multiplies after, the other before the butterfly |
| Bit-reversed order | index order obtained by reversing the binary digits of each index (e.g. 1 = 001 → 100 = 4 in 3 bits) |
| Streaming | data flows through fixed pipeline stages continuously, instead of being processed as whole matrices |
| RDA | Range-Doppler Algorithm, the SAR focusing method used here |

## 9. One-paragraph summary

init_param.m simulates echoes from two point targets and prepares one range filter plus one azimuth filter per range row. main.m focuses the data two ways: the classic way, with MATLAB's fft/ifft on columns then rows, and the streaming way, where the azimuth transform is built from 9 DIF stages followed (after the filter multiply) by 9 DIT stages. Chaining DIF then DIT removes the need for bit-reversal reordering memory, at the cost of a bit-reversed azimuth filter (H_az2). The final DIT stage as written is a forward FFT rather than an inverse one, so its output is scaled by 512 and index-reversed in azimuth. Check this when you compare the two plots.